User Manual for install COREDRAW

1. Install COREDRAW application software.

2. Click Start to choose the equipment—printer, double-click add printer ico enter into the guide of add printer:

Click Next;

Choose local printer, then click Next (if no LAN, then no use to do this step);

Choose Install from the disk:

In the “copy manufactory file from” Choose OPS628v712\WIN WIN2K_XP （choose this drive in WINDOW 2000 or XP）, and choose Graphtec CE1000-60 in the printer(P), then click Next until Finish.

3. Set up the Graphtec CE1000-60 to Default Value, enter into the Properties of Graphtec CE1000-60 to amend Paper parameter;

Set up paper size to Free size;

Amend Plot area to: length of materials*the cutting width of plotter.

Choose Landscape in orientation.

The landscape orientation in CORELDRAW is the portrait in plotter.

Must choose “Grayscale”

Ensure the option is same as the above picture, otherwise will appear scrawl

Remark:

- In every cutting output, pitch on the export ico, use the contour tool to make the outline, then use the fill in tool to do the no fill in step.

- In the export printer dialog, set the layout and let the ico in the lower left, then output printing.